# Rensselaer Mechatronics

# Lab 1 submission

- Tutorial Screenshot.  Paste the screenshot described at the end of the tutorial that shows your Matlab, Simulink, code, path, figure and workspace:

- Euler’s Method. Consider the following ordinary differential equation (ODE):

4x+3x-2=0

with initial condition, x0=2. Using a step size of ∆t=0.5, numerically approximate the solution, xt, up to t=1.5 using Euler’s method. Sketch a plot of the solution by hand; show all work.

- Numerical Solution with Simulink. The following ODEs describes the dynamics of a freight train using a point-mass model:

d(t)=v(t)

vt=1MP(t)v(t)-A+Bvt+Cv2t

where d(t) is the distance along the route, vt is the velocity of the train, and P(t) is the power from the locomotive. The polynomial of vt with coefficients A, B, and C models the resistance to motion from different factors such as bearing resistance, rolling friction, and aerodynamic drag. The total mass of the train is M.

The following parameters are given:

M=2200 tons =2×106kg

A=40 N

B=2 N∙s/m

C=20 N∙s2/m2

Consider a 10-kilometer segment of the trip, with initial conditions d0=0,  v0=45 km per hour. The goal is to determine a constant power setting for that segment that allows the train to complete the segment in the shortest amount of time without exceeding the track speed limit of 80 kilometers per hour. (Be careful with the velocity units in the simulation!)

Locomotives have discrete power settings, called “notches.” You will test two notch powers to determine the one that meets the objectives:

Notch 3 : 536 horsepower =4×105 Watts

Notch 4 : 804 horsepower =6×105Watts

You will use Simulink to solve the ODE required to simulate the train motion with the different power settings. Simulink uses a built-in numerical process similar to Euler’s method to approximate the solution. Follow the process described in the tutorial that you completed in class. Obtain data every 1 second for a duration of 15 minutes.

  - Draw the simulation diagram by hand.  Paste the hand drawn diagram below:

  - Create a Simulink diagram as you did in the tutorial. Note that Simulink often has several ways to do different mathematical operations. E.g. in this problem, the “polynomial” block may be useful. Provide a screenshot of the Simulink simulation diagram:

  - Write an m-file to run the simulations as described in the tutorial. Provide a screenshot of the m-file script.

  - Provide a screenshot of the graph generated by the Simulink file and m-file script for each of the two power settings. For this problem, each plot should include distance, dt, and velocity, vt, versus time. Label the graph properly as described in the course syllabus.

  - Which notch setting meets the objectives for this segment of the trip? Quantify the answer in terms of time to reach a distance of 10 km and maximum speed during that time.