# Control Systems Lab

# Lab 2 In-Class Exercise

# Part 1: Identifying transfer function parameters from step response

- Motor ID: The DC motor system and Multimeter are labeled with a number. Please make note of them in Gradescope.

- Step Response Data: Obtain the step response data for the motor by simply running ‘M2V5_Motor_Step_Response_CSL.slx’ on the motor system.  You do not have to modify the file at all - just open, examine the code so you know what experiment it is performing, run the code, and stop it after it has collected several steps. Save the data to a .mat file and upload the .mat file to Gradescope.

- Step Response Plot: Plot the data for only one single step that goes from one NON-ZERO steady state speed to another steady state speed. The plot should clearly show one and ONLY one step including the transient portion of the response: (use the xlim, ylim commands or the graph zoom tool introduced in Lab 1.5)

This is the step response you will determine K and τ from in the lab submission.  Upload a screenshot of a properly annotated plot in Gradescope (Upload a .png. NOT the .fig file and NOT a .pdf file – make sure you can see the actual plot in gradescope).

# Part 2: Identifying ODE parameters through direct measurements

- Motor Resistance: You will estimate the average resistance of the motor by holding the disk at zero speed and measuring the voltage across it, and current flowing through it, using a multimeter.   Use your prelab voltages for the Specified voltage.

  - Use your prelab voltages for the Specified voltage of each experiment in the table.

  - Open ‘M2V5_ParameterID_CSL.slx’. Set your first voltage by changing the value in the ‘Voltage command’ block in the Simulink diagram. Press ‘Monitor and Tune’.

  - Once the motor starts to turn, hold the motor with your hand so it is not spinning

  - Measure voltage and current, referring to ‘Voltage_Current_Measurements’ for specific instructions on using the multimeter.

| Specified voltage: Vs [V] | Measured voltage: Vm [V] | Measured current: Im[Amp] | Resistance: Rm (Ω) |
| --- | --- | --- | --- |
|  | Average Resistance: Ravg (Ω) |  |  |

  - Fill the table above and compute Resistance, Rm (Ω), then calculate the algebraic mean of the Resistance values, Ravg (Ω). How does it compare with the nominal motor resistance given in the Prelab, 7.22Ω ?  (you do not need to answer this in Gradescope, but if it is significantly different contact your instructor).

  - Upload a screenshot of the completed Table into Gradescope.

- Back EMF constant: You will estimate the average Back-EMF Constant, kb (V.s/rad) of the motor, when it is turning freely, by measuring current, voltage, and speed.

Use your prelab voltages for the Specified voltage.

  - Continue running, or start, the Simulink diagram from the previous part.

  - Measure current and voltage while the disk is spinning.

  - Measure speed by double-clicking the Scope and estimating the average visually.

  - Use the Average Resistance value that you calculated in step 3 and the steady state electrical equation to calculate the value of the Back EMF Constant,  kb, completing the table below.

  - Upload a screenshot of the completed Table into Gradescope.

| specified voltage: Vs [V] | measured voltage: Vm [V] | measured current: Im [Amp] | measured speed: ωm [rad/s] | kb [V.s/rad] |
| --- | --- | --- | --- | --- |
|  | Average Back EMF-Constant: kb.avg [V.s/rad] |  |  |  |