# Control Systems Lab		         Name:________________

# Prelab 3: In Class Assignment	SEC:____RIN:_____________

- Plotted below are an input voltage and an output speed for a DC motor:

  - What is the AMPLITUDE of the INPUT signal (include units)?__________________________

  - What is the FREQUENCY of the INPUT signal (in radians per second)?

- What is the OFFSET of this signal (include units)?________________________________

  - Why do we include an offset in the input voltage for the experiments?

- Consider the transfer function between the input and output above

  - Estimate the MAGNITUDE of the transfer function from the plot

  - Estimate the PHASE of the transfer function from the plot

- Below is a typical bode diagram for the CSL DC motor system:

From the bode diagram extract the PREDICTED magnitude and phase for an input frequency of 30 rad/s (use units of rad/secvolt, and degrees):

Mag = ____________________ 		Phase = ______________________

- In the Lab experiment, you will measure magnitude and phase (similar to problem 3). You will compare this to predictions from the bode plot (similar to problem 4). Based on inspecting the predicted bode diagram above, and using your own intuition, propose a list of test frequencies. Note there is not a single, exact, right answer, but the frequencies should be appropriate to cover the range of interest. Mark the frequencies on the diagram above, and list them below: