# Control Systems Lab

# Lab 5 In-Class: Position Control

# Objectives

- Experimentally test proportional controls gains that were determined from model-based design

- Quantify actual position control performance in terms of steady state error and overshoot

# Overview

In this lab you will test the proportional gains that you designed in the prelab. You will be required to apply tools and experience learned in previous labs, so not all methods will be explicitly described in the instructions. For example, you will need to modify a Simulink diagram, and analyze and plot data.

# Experiments

- Using the posted Lab 5 Simulink diagram assemble the block diagram for a proportional controller.

Verify your Simulink diagram does the following:

  - Uses the potentiometer as a reference value – make sure you convert it to radians (use the value you calculated at the end of lab 4 or the value from prelab 5)

  - Computes the voltage input to the motor using proportional control

  - Views and logs both the reference and the position outputs

    - Add to workspace blocks (remember to use “2d array” and not “timeseries”)

Put your name in the white space of the Simulink diagram.  Upload an image or screenshot of your Simulink diagram to Gradescope. (double left click in the Simulink diagram white space and select ‘create annotation’).

- General observations from increasing the gain when your control code is running:

Set proportional gain of zero, then perform the following experiments:

- Use the potentiometer to change the reference position by approximately π radians and move it back and forth and observe the response in both directions.

- Another way to observe performance is to apply a disturbance.  Move the motor wheel directly by hand half a turn, release it and see how well it recovers after you let go.

Repeat these experiments as you increase the value of the proportional gain until it becomes unstable (around 20 times the maximum Kmax calculated in Prelab 5).

Briefly describe how speed, steady state error and the overshoot/stability change as you increased the gain from 0 to the max value you tried. Upload these observations to Gradescope.

- Test performance with gain designed for critical damping (Kcrit) and for 10% overshoot (K10)

Save this data!:

  - Now, in the Simulink diagram, replace the potentiometer input with a constant block to represent the reference. Use this to specify a reference position of 8π and back to zero (repeat a couple of times changing the value in the constant block while it is running) and run experiments with Kcrit and K10 from Prelab 5.  Graph the responses to the two gains on separate plots, including both the reference and the actual response on each plot. Upload images of your graphs to Gradescope.  Do not use the scope, but plot data saved to the workspace so you can properly annotate the graph.

  - Upload your two data .mat files of each experiment to Gradescope.

- Test performance with maximum gain (Save this data!):

  - Consider the maximum gain, Kmax, to avoid saturation with a reference position of 1.5π, as calculated in the Prelab 5. Run this experiment. Graph the response, including both the reference and actual position.

  - Upload your .mat file to Gradescope.